ChronoBloom Compact Function Inventory

Print map - common edit targets first; support internals condensed. Source SHA256: 52d8af3a...c498b

Common / Reasonably Likely Edit Targets

| Function / structure | Telegraph meaning | Edit when |
| --- | --- | --- |
| SettingsStore::defaults | Factory defaults | Default palette, brightness, feature defaults. |
| SettingsStore::valid | EEPROM accept rules | Change when fields/ranges change. |
| SettingsStore::sanitize | Clamp user input | Safety bounds for saved settings. |
| ClockSettings | Saved settings schema | Add field -> bump SETTINGS_VERSION. |
| RingConfig | Physical LED map | Ring start index, count, direction. |
| ClockRenderer::render | Full frame pipeline | Overall draw order/priority. |
| ClockRenderer::renderFace | Static ring backdrop | Markers, filler, base colors. |
| ClockRenderer::renderSeconds | Second hand logic | Trail/progress/dot behavior. |
| ClockRenderer::renderMinutes | Minute dot | Minute placement/color behavior. |
| ClockRenderer::renderHours | Hour hand logic | 12h mapping, half-hour advance. |
| ClockRenderer::effectiveBrightness | Brightness selector | Manual/lux/scheduled dimming. |
| ClockRenderer::nightActive | Night window test | Scheduled dimming hours. |
| ClockRenderer::setRingPixel | Logical -> physical LED | Rotation, offset, direction fixes. |
| ClockRenderer::setCenterPixel | Center LED writer | Inline vs separate center output. |
| ClockRenderer::setSacrificialPixelDark | Keep buffer LED black | Sacrificial first LED handling. |
| ClockRenderer::renderCenterIdle | Center breathing pulse | Idle center animation style. |
| ClockRenderer::renderStatus | Status spinner/pulse | WiFi/save/error visual feedback. |
| ClockRenderer::renderHourlyChime | Top-hour sweep | Hourly chime visual style. |
| ClockRenderer::tickAnimation | Animation state machine | Quarter/half/hour effects. |
| triggerQuarter/Half/HourAnimation | Start selected animation | Selector mapping and no-op rules. |
| LuxSensor::lux | Read + smooth lux | Gain thresholds, EMA, settle time. |
| LuxSensor::autoBrightnessCached | Lux -> brightness ramp | Curve, min/max feel, ramp speed. |
| TimeSync::begin | NTP/TZ startup | Timezone and NTP servers. |
| TimeSync::syncNow | Apply valid NTP time | Epoch guard and localtime conversion. |
| TimeSync::loop | Periodic resync | Callback + 6-hour refresh behavior. |
| TimeModel::set/addMinutes/addHours | Manual time edits | Browser time buttons and sync. |
| WebUi::begin | WiFi/web bringup | AP fallback, mDNS, OTA, NTP order. |
| WebUi::setupRoutes | HTTP route table | Add/change browser endpoints. |
| WebUi::settingsJson | Settings API output | Expose new settings to UI/JS. |
| WebUi::htmlPage | On-device web UI | Labels, controls, JS preview. |
| WebUi::parseColor | Hex color parser | Color input validation behavior. |
| WebUi::clampByte/clampWord | POST bounds guards | Form min/max safety. |
| FocusReminderScheduler::checkAndFire | Reminder gatekeeper | Days, hours, interval firing. |
| FocusReminderScheduler::getDayOfWeek | Weekday source | Needs pre-NTP epoch guard. |
| FocusReminderScheduler::triggerReminderAnimation | Reminder animation map | Map options to effects. |
| setup | Boot sequence | Add subsystem init here. |
| loop | Main scheduler | Time tick, web, render priority. |
| logRuntimeStatus | Serial health dump | Debug fields and cadence. |

ChronoBloom Compact Function Inventory - Back Side

Rules That Prevent Pain

| Rule | Why it matters |
| --- | --- |
| EEPROM rule | Changing ClockSettings layout requires SETTINGS_VERSION bump or stale EEPROM will bite. |
| Rotation rule | outerRingOffset is one outer-ring step; if one step worse, reverse direction. |
| Sacrificial LED rule | Do not remove setSacrificialPixelDark; buffer LED must stay black. |
| UI setting rule | New setting usually touches ClockSettings, defaults, valid, sanitize, settingsJson, setupRoutes/htmlPage. |
| Animation rule | Visual look lives mostly in tickAnimation; selector start lives in trigger*Animation. |

Fast Edit Path Cheat Sheet

| Problem / change | Start here |
| --- | --- |
| Color/default look | SettingsStore::defaults, renderFace, renderSeconds/Minutes/Hours |
| Clock physically rotated | setRingPixel, RingConfig, outerRingOffset |
| Sacrificial WS2812B | setSacrificialPixelDark, ring offsets/config, pixel count |
| Web UI change | htmlPage, setupRoutes, settingsJson, clamp/sanitize |
| New saved option | ClockSettings, defaults, valid, sanitize, settingsJson, POST handler, SETTINGS_VERSION |
| Brightness feels wrong | effectiveBrightness, LuxSensor::lux, autoBrightnessCached |
| Reminder weirdness | checkAndFire, getDayOfWeek, triggerReminderAnimation |

Rarely Edited / Support Internals

| Internal plumbing | Internal plumbing |
| --- | --- |
| configureWiFiHostname - hostname plumbing before DHCP | ClockRenderer::begin - strip init/clear/show |
| writeStatusLed - GPIO LED helper; no-op when STATUS_LED_PIN < 0 | ClockRenderer::renderAnimFrame - animation render wrapper |
| setupWiFi - usually edit credentials/build flags, not logic | ClockRenderer::needsFullAnimationFrame / needsCenterAnimationFrame - 80 ms render gates |
| setupOTA - OTA handlers/password/log text | ClockRenderer::dim/scale/pulse - color math helpers |
| LuxSensor::begin - VEML7700 init only | ClockRenderer::chimeActive/statusActive/centerIdleActive - small boolean gates |
| LuxSensor::autoBrightness - wrapper only | TemperatureInput - disabled stub in current builds |
| LuxSensor::autoBrightnessTarget - diagnostics only | WebUi::loop - handleClient + ArduinoOTA pump |
| SettingsStore::begin/update - EEPROM load/save wrapper | ClockTime - h/m/s POD copy |
| TimeModel::get/tickOneSecond/consumeDirty/incrementSecondNoLock - clock plumbing | AnimPhase / StatusMode - enum IDs for animations/status |

Calibration note: outerRingOffset direction depends physical wiring. Move one step, test, then reverse if worse. No heroics - LEDs punish confidence.